Workstation Case Study: Workbench

| ErgoSystems Workstation Design Worksheet |  |  |  |
| --- | --- | --- | --- |
| Job/Task: Assembly M3057 Unit | Date: 12-19-08 | Time: 1:00 PM | Analyzed by: Mark Anderson |
| Area/Dept/Location: 70445 | Project Number: 100345 |  | Employee Name: Jane Smith |
| Supervisor: Marge Jones | Workstation: M3057 Unit |  | Job Title: Equipment Builder |

Define Design Question

Define Design Question with necessary detail to accurately develop Design Specifications.

Workbench Define the Design Specifications for a height adjustable workbench. Specifications to include: - Workbench height: Low/High adjustment range as measured from top of the worksurface to the floor. - Workbench dimensions: Worksurface depth, width and thickness. - Workbench foot/leg clearance: Specifications to allow for adequate foot/leg clearance under the worksurface. - Footrest platform: Size and location for footrest (if needed). - Method of height adjustment: Powered (type) or manual crank. - Storage bin location: Height and reach to bins (in needed). - Workbench weight capacity: Load capacity requirement for workbench.

| Develop Design Criteria |  |  |
| --- | --- | --- |
| Develop the Design Criteria using the W5H Approach (Who, What, When, Where, Why and How). |  |  |
| W5H | Questions | Outcome |
| Why Why will the workbench be used? | Validate rationale for workbench: - Is the workbench the best approach? - Modify process to eliminate need for the workbench? - Incorporate process at another location? - Use other type of workstation? (e.g. cart) | Use of workbench validated: - Use of the workbench determined to be value added and integral to the process |
| Who Who will work at the workbench? | Define user population:  Description of the user population? (size, stature, age, experience, handedness, etc.) | Diverse user population: - Small female to large male - Need to include 5th percentile female (5’0”) to 95th percentile male (6’2”) - Age range from 18 to 75 - Wide range of experience - 90% right handed, 10% left handed |
| What What will be worked on at the workbench? | Define/describe parts, materials, tools, etc.:  Size, shape, weight, quantity?  How staged or stored?  Influence appropriate workbench height? | Parts: - Parts staged on the bench- Work in Progress (WIP) of 3 - Small parts stored in bins at workbench - Need to include 5th percentile female reach zone - Parts up to 35# in weight - Parts up to 5” in height - Parts handled from bottom of part - Parts up to 12” in depth - Parts up to 36” in length Tools: - Hand tools (screwdriver) - Counterbalanced overhead inline driver (air powered) |
| How How will the workbench be used? | Describe how workbench will be used:  Standing or seated worker position?  Describe job task (precision, light assembly, forceful assembly/manual handling).  Task will influence appropriate workbench height. | Position: - Standing worker position Job Task: - Moderate forceful assembly with manual handling - Indicates work level/hand position at 2 to 6” below elbow level |
| When When will the workbench be used? | Determine timeframe of use of workbench:  Intermittent or continuous use?  Indication of exposure of user to the workbench | Continuous use: - Three shifts - High workforce exposure to workbench |
| Where Where will the workbench be used? | Determine location of workbench:  Stand-alone bench or integrated into a line?  If integrated, how will it fit with other workbenches?  How will parts be conveyed from workbench to workbench?  Adequate access to workbench – ingress/egress? | Stand alone workbench: - Small parts in bins at bench - Large parts conveyed to workbench via cart |

| Generate Design Specifications |  |  |
| --- | --- | --- |
| Generate the Design Specifications making use of the appropriate databases (ANSUR Database and\or other databases as indicated). |  |  |
| Summarize Design Criteria |  |  |
| Validation | Use of workbench is validated. |  |
| Diverse user population | Population includes 5th percentile female to 95th percentile male. |  |
| Parts | Parts weigh up to 35# and sizes up to 5” by 12” by 36”, small parts stored in bins at workbench. |  |
| Tools | Hand tools and overhead counterbalanced inline driver. |  |
| Work position | Standing work position. |  |
| Job Task | Moderate forceful assembly with manual handling. (Indicates work level/hand position at 2” to 6” below elbow level) Bin storage within reach of 5th percentile female |  |
| Determine required data point(s) and results |  |  |
| Standing | Elbow Height (5th%tile Female=37.6”, 95th%tile Male=47.1”) |  |
| Standing | Forward Functional Reach- Acromial Process to Functional Pinch (5th%tile Female=22.0) |  |
| Standing | Shoulder Height (5th%tile Female=46.5”, 95th%tile Male=61.4”) |  |
| Standing | Eye Height (5th%tile Female=55.2”, 95th%tile Male=69.5”) |  |
| Footrest Location | Location - Height from floor with toe clearance (6” footrest height, 5” in from workbench edge) |  |
| Knee Clearance | Clearance (5” from edge of workbench) |  |
| State Design Specifications |  |  |
| Workbench height: Measured from top of the worksurface to the floor. Calculated by subtracting 4” from min/max values. |  | 34” to 44” (Low/High adjustment range) |
| Method of height adjustment: Powered (type) or manual crank. |  | Powered (Electric-pneumatic height adjustment) |
| Storage bin location: Height and reach to bins. Note: attached to workbench and will move up/down in sync with bench. |  | 10” to 20” (Above workbench surface, allows reach to head level) 22” (Maximum reach to back of bin from front edge of workbench) |
| Workbench dimensions: Worksurface depth, width and thickness. |  | Depth: 30” Width: at least 48” Thickness: max of 2” |
| Workbench foot/leg clearance: Specifications to allow for adequate foot/leg clearance under the worksurface. |  | 5” (From edge of workbench) |
| Footrest platform: Size and location for footrest (if needed). |  | 4” to 6” (Height from foot support surface and depth from front of any obstruction.) |
| Workbench weight capacity: Weight capacity based on parts, tools and table top weight (stainless steel). |  | 500# |

| Test Design Specifications |  |
| --- | --- |
| Test Design Specifications by applying to appropriate cases in the workplace. Modify design as indicated. |  |
| Mock-up design as needed to allow testing. Identify individuals who represent Design Specification boundaries. Perform test and modify design as indicated. | Accomplished: workbench mocked up and tested. Design criteria were met. |

| Implement Design Specifications |  |
| --- | --- |
| Implement the Design Specifications and follow-up once in place. |  |
| Implement the Design Specifications. | Accomplished: workbench in place and operational. |
| Follow-up to ensure adequate user acceptance. | On-going. |

Comments